DIGITAL VLSI SOC DESIGN AND PLANNING

THEORY:

- Took the Arduino board as an example, to demonstrate the SoC /processor.

- The interfaces around the processor are JTAP to program the processor, I2C-EEPROM,QSPI1-Flash,SDRAM,UART and GPIOs.

View of package

Packages given with the name of QFN-48 – Quad Flat No Leads(there are different other packages).

Chip is sitting at the centre of package.

Chip is connected to pins using wirebound

What is inside chip?

- Chip has various components, one of the important component is PADS.

- Pads are something, through which you can send signal inside the chip.

- Core of the chip, is where all the digital logic sits.

- Die is the area of the chip. It is manufactured on a silicon wafer.

Example of RISC V SoC:

- Typical core area consists of SoC, SPI,PLL,SRAM,ADC,DAC etc,

- PLL,ADC,DAC,SRAM are called foundry IP’s.

- SoC, SPI are called macros.

RISC V Instruction set architecture(ISA):

- Implement RISC V specification using RTL(picorv32 cpu core).

- Synthesized netlist of RTL.

- Physical design implementation of netlist.

- RTL to layout is the standard RTL2GDSII flow.

How do apps run on hardware?

- Application software enters into system software, system software consists of OS, Compiler, Assembler.

- This OS will take this apps and converts it into assembly level and then to binary or machine level language.

- Compiler takes C,C++,java (high level code) and convert it into set of instructions(.exe).

- Assembler takes this instructions and convert it into binaries (0,1s).

- These instruction set here are called as RISC V architecture.

- Interface used to reach from instructions to machine level is hardware description language(hdl).

SoC Design Using OpenLANE

- We need RTL IP’s, EDA Tools, PDK Data to design the ASIC.

- Open source EDA Tools, RTL designs, PDK data  are used.

- PDK = the interface between the fab and the designers.

- PDK contains, process design rules(DRC,LVS,PEX), device model, digital standard cell libraries, I/O libraries.

ASIC Design Flow:

- ASIC flow objective: RTL to GDSII( Also called automated PnR and/or physical implementation.

Simplified RTL to GDSII Flow:

1.Synthesis:converts RTL to a circuit out of components from the standard cell library(scl).

- Standard cells-have regular layout.

2.Floor/power planning: plan the silicon area and to create robust power distribution to power the network.

  - Chip floor planning: partion the chip die between different system building blocks and place the I/O pads.

  - Macro floor planning:dimensions, pin locations, rows definition.

  - Power planning: chip is powered by multiple vdd and gnd pins. They are connected through vertical and horizontal metal straps.

3.placement : place the cells on the floorplan rows, aligned with the sites.

- 2 steps: global(not legalled) and detailed placements(legalled).

4. clock tree synthesis : create a clock distribution network to deliver the clock to all cells with minimum skew(0 is hard to achieve)

- Usually a tree- H,x,pi tree

5.routing: implement the interconnect using the available metal layers.

6.signoff :

- Physical verifications: design rules checking (DRC) and layout vs schematic(LVS).

- Timing verifications: static timing analysis(STA).

OpenLANE ASIC flow:

- Produce a clean GDSII with no human intervention (no-human-in-the-loop).

- Tuned for skywater130nm open PDK.

- Can be used to harden macros and chips.

- OpenLANE is based on several open source projects – openroad,fault,yosys,abc,qflow,magic .

- DFT to scan insersion(JTAG)

- Physical implementation also called automated PnR(Place and route)

- LEC(logic equvilence check): everytime the netlist is modified, verification must be performed.

- Dealing with antenna rules violations:

1. Bridging attaches a higher layer intermediary

2. Add antenna diode cell to leak away charges.

- Physical verification: DRC(magic) and LVS(magic and netgen).